********************************************************************************

*** Extra Flyer Calling Docs                                     23 Aug 1994 ***

********************************************************************************

**** Description of the ClipAction structure ****

STRUCTURE ClipAction,0

APTR	ca_Volume	Pointer to a FlyerVolume structure (see below)

APTR	ca_ID		References Flyer operations going on

asynchronously.  Can be pulled out and stored,

then re-inserted into a ClipAction structure

which is then passed to WaitAction, AbortAction,

or CheckAction.  When the operation is complete,

some results can be found in the ClipAction

structure as well.

UBYTE	ca_ReturnTime	Specifies when call should return.  See RT_xxx

values.  Note that some calls cannot return until

complete (these will ignore this field).  Default

value is RTT_STOPPED.

UBYTE	ca_Channel	Which hardware video channel to use.

Channel #	Hardware

---------	-------------------------------

0		Flyer output A (Toaster input 3)

1		Flyer output B (Toaster input 4)

Default value is channel 0.

UBYTE	ca_Flags	Various control flags:

CAF_VIDEO	Include video in action

CAF_AUDIOL	Include left audio in action

CAF_AUDIOR	Include right audio in action

CAF_USEMATTE	After play complete, display matte

color given in MatteY,I,Q fields

CAF_NOPREROLL	OBSOLETE

CAF_APPEND	OBSOLETE -- use AppendFields() function

CAF_REPROCESS	Skip optimizations for CutList operations

Default value is CAF_VIDEO!CAF_AUDIOL!CAF_AUDIOR

UBYTE	ca_PermissFlags		Various permission flags:

CAPF_STEALOURVIDEO

Can steal requested video channel in

order to successfully perform new

action specified.

CAPF_KILLOTHERVIDEO

Can kill video on other channel(s) if

needed to successfully perform new

action specified.

CAPF_ERRIFBUSY

If set, will return an error if the

Flyer cannot perform the specified

action immediately.  If not set, will

wait for needed resources/channels to

free up and then will begin action.

CAPF_AUTOMUTE

With DoFindField, mute audio after played once

CAPF_USEHEADS	OBSOLETE

ULONG	ca_VidStartField

ULONG	ca_AudStartField

When playing a clip, these values specify the start field number

at which to begin (for video and audio).  Think of this as an offset

field number "into the clip".  The first field number of a recorded

clip is always 0.  Default = 0.

ULONG	ca_VidFieldCount

ULONG	ca_AudFieldCount

These specify limits on the number of fields to play or record,

for both audio and video.  A value of 0 means "unlimited".

Default = 0.

ULONG	ca_GoClock	Specifies the field clock value (the Flyer's) at which to

begin playing or recording.  A value of 0 means ASAP.

Default value is 0.

UWORD	ca_MatteY	Define the matte color Flyer will put up when a clip has

BYTE	ca_MatteI	finished playing (if CAF_USEMATTE set) -- Only BLACK yet!

BYTE	ca_MatteQ	Also used with SetFillColor() -- all values supported

UWORD	ca_VolAttack	Attack time of the volume envelope in fields

Default value is 0.

UWORD	ca_VolSust1

UWORD	ca_VolSust2	These specify the "plateau" level of the volume envelope

(for each audio channel).  $0000 is softest, $FFFF is

loudest.

Default value is $FFFF.

UWORD	ca_VolDecay	Decay time of the volume envelope in fields

Default value is 0.

WORD	ca_AudioPan1

WORD	ca_AudioPan2	Pan position for audio

-$8000 is full left

$0000 is center

$7FFF is full right

All intermediate values supported too.  Default is 0.

ULONG	ca_TotalAudStart	OBSOLETE

ULONG	ca_TotalAudLength	OBSOLETE

ULONG	ca_StartBlk	If no name is specified in the "fv_Path" string, this value

specifies a "raw" block number at which to start a clip

playing or recording.  This field is also updated with the

actual block number found after a call that locates a

specific field.

Default value is 0.

ULONG	ca_EndBlk	If no name is specified in the "fv_Path" string, this value

specifies a "raw" block number at which to stop playing

or recording.

Default value is 0.

ULONG	ca_Grip		\

ULONG	ca_FileID	 These are used privately by the FlyerFileSystem

UBYTE	ca_Access	/

ULONG	ca_FldHandle	Reference number for an open field for R/W operations.

Set for you by OpenReadField/OpenWriteField calls and

used by FlyerWriteLine/FlyerReadLine/CloseField calls.

ULONG	ca_LastFieldDone

Indicates the last field number played or recorded.  Valid

only when the action is complete (stopped or was aborted).

For recording, this value = number of fields recorded - 1

For playing, this value will depend on the starting field

number and how many fields were done.

**** Description of the FlyerVolume structure ****

STRUCTURE FlyerVolume,0

APTR	fv_Path		Pointer to a null-terminated string which contains

a volume name, clip name, or both.  If ptr is NULL,

or if no volume is specified, then next three values

must be set properly to specify the Flyer card to

talk to and the exact SCSI channel & drive numbers.

Default value is NULL.

UBYTE	fv_Board	Flyer board number (0...n).  Not used if volume

is specified in fv_Path string.  Default = 0.

UBYTE	fv_SCSIdrive	SCSI drive number.  This is a combination of the

channel number and unit number, and ranges from 0 to 23.

Usually filled in for caller if volume is specified in fv_Path

string.  Default = 0.

UBYTE	fv_Flags

FVF_USENUMS - Specifies that fv_SCSIdrive number shall override

any volume found in the fv_Path string.