Using the Horita TRG-50PC with the Flyer.

By

Jeff Scheetz & LLoyd Slapar

As of 10/11/95 NewTek Tek Support

Expectations-

1.- The New Tek Video Flyer in revision 4.07 (and several before) does

support the use of  Drop Frame Time Code.

2.- The TRG-50 Supports LTC or Longitudinal Time Code.  LTC is generally

read from a third audio track on beta VTR's.

3.- If  your specific application denamds use of  VITC or Vertical Interval

Time Code you will to add a Horita VTL-50 to your system. The VTL-50 reads

the VITC and puts out an LTC signal that the TRG-50 can understand.

Additional Required Hardware:

1.  Null modem adaptor (Radio Shack. Female DB9 to Male DB9 Cat. No. 26-264A)

2.  75 ohm BNC terminator also available from Radio Shack.

Connections:

To record LTC to the Flyer-

1. Connect the TC OUT  (or AUDIO 3 out) of your VTR to the TC IN on the TRG-50.

2. Connect REFERENCE VIDEO OUT (or Sync out) to the VIDEO IN of the TRG-50.

3. Connect the VIDEO OUT of  your VTR into the INPUT 1 of the Flyer directly.

4. Place a 75 ohm terminator on the TRG-50's VIDEO OUT  to terminate the

reference signal.

5. Connect the TRG-50's COMM  cable from the TRG-50 to the NULL MODEM ADAPTER.

6. Connect the NULL MODEM ADAPTER to the Flyer's SERIAL A cable, and double

check the other end of the SERIAL A cable to be certain that it's connected

to SCSI A on the Flyer.

7. Be sure the TGR-50 is receiving power.

8. The TRG-50 TC out can be left empty.

TRG-50 Settings:

Data should be set to TC.

Mode should be set to RDR.

V-SIZE and V-POS refer to window placement and can be ignored.

Operation:

Same procedure as always.

FYI:

The Flyer will not display the Time Code while recording.

Once in the MAKE CLIPS window, the default IN and OUT points should reflect

the Time Code numbers.

The Flyer will not playback time code or output an Edit Decision List.

However EDL output may be handled by out third party developers.

The Horita outputs serial data as SMPTE 80-bit longitudinal drop- or

non-drop frame code.  Drop-frame code is indicated by semicolon separating

seconds and frames values.  Non-drop-frame code is indicated by a colon

separating seconds and frames values.  Frame 1 is indicated by a period

separating seconds and frames.

Null Modem Notes:

For those who wish to make a null modem, the pin connections are as follows.

Female DB9 1 	2	3	4	5	6	7	8	9

Male DB9   4	3	2	6 &1	5	4	8	7	NC

Additional Information: Call

NewTek

Tek Support

(913) 228-8282

or

Horita

(714) 489-0240