TABLE OF CONTENTS

flyer.library/AbortAction

flyer.library/AddClipCut

flyer.library/AddSeqClip

flyer.library/AppendFields

flyer.library/BeginFindField

flyer.library/ChangeAudio

flyer.library/CheckAction

flyer.library/CloseField

flyer.library/CopyData

flyer.library/CPUDMA

flyer.library/Defaults

flyer.library/DoFindField

flyer.library/EasyOpenWriteField

flyer.library/EndClipCutList

flyer.library/EndFindField

flyer.library/EndHeadList

flyer.library/EndSequence

flyer.library/EndSequenceNew

flyer.library/Error2String

flyer.library/FindDrives

flyer.library/FindFieldAudio

flyer.library/Firmware

flyer.library/FlyerAudioCtrl

flyer.library/FlyerCopyClip

flyer.library/FlyerCopyClipNew

flyer.library/FlyerCreateDir

flyer.library/FlyerDeFrag

flyer.library/FlyerDeFragNew

flyer.library/FlyerDelete

flyer.library/FlyerDirList

flyer.library/FlyerDriveCheck

flyer.library/FlyerDriveInfo

flyer.library/FlyerFileClose

flyer.library/FlyerFileOpen

flyer.library/FlyerFileRead

flyer.library/FlyerFileSeek

flyer.library/FlyerFileWrite

flyer.library/FlyerFormat

flyer.library/FlyerInputSel

flyer.library/FlyerPlay

flyer.library/FlyerQuit

flyer.library/FlyerReadCalib

flyer.library/FlyerReadLine

flyer.library/FlyerRecord

flyer.library/FlyerRename

flyer.library/FlyerRenameDisk

flyer.library/FlyerRunning

flyer.library/FlyerSetBits

flyer.library/FlyerSetComment

flyer.library/FlyerSetDate

flyer.library/FlyerStripAudio

flyer.library/FlyerTermination

flyer.library/FlyerWriteCalib

flyer.library/FlyerWriteLine

flyer.library/GetClipInfo

flyer.library/GetFieldClock

flyer.library/GetFrameHeader

flyer.library/GetSMPTE

flyer.library/InitFlyers

flyer.library/Inquiry

flyer.library/LocateField

flyer.library/LockFlyVolList

flyer.library/MakeClipHead

flyer.library/MakeFlyerFile

flyer.library/ModeSelect

flyer.library/ModeSense

flyer.library/NewSequence

flyer.library/OpenReadField

flyer.library/OpenWriteField

flyer.library/PauseAction

flyer.library/PlayMode

flyer.library/PlaySequence

flyer.library/PutFrameHeader

flyer.library/Read10

flyer.library/ReadSize

flyer.library/ReadTest

flyer.library/RecordMode

flyer.library/ReqSense

flyer.library/ResetFlyer

flyer.library/SCSIinit

flyer.library/SCSIreset

flyer.library/SCSIseek

flyer.library/SetFillColor

flyer.library/SetFlooby

flyer.library/SetFlyerTime

flyer.library/SetSerDevice

flyer.library/SkipLines

flyer.library/StartClipCutList

flyer.library/StartHeadList

flyer.library/StillMode

flyer.library/TBCcontrol

flyer.library/ToasterMux

flyer.library/UnLockFlyVolList

flyer.library/VideoCompressModes

flyer.library/VideoParams

flyer.library/VoidAllHeads

flyer.library/VoidCardHeads

flyer.library/VoidClipHead

flyer.library/WaitAction

flyer.library/Write10

flyer.library/WriteTest

flyer.library/AbortAction                           flyer.library/AbortAction

NAME

AbortAction - abort a previously started action

SYNOPSIS

error = AbortAction(action)

D0                  A0

ULONG AbortAction(struct ClipAction *);

FUNCTION

Attempts to abort an action that was previously initiated.  Does

nothing if it has already finished.

If ClipAction ptr is NULL, aborts all pending operations to all

Flyers.

INPUTS

action - ptr to ClipAction structure used to start the action

(or NULL to abort everything)

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/AddClipCut                             flyer.library/AddClipCut

NAME

AddClipCut -- Add an entry to the ClipCut list

SYNOPSIS

error = AddClipCut(subclip)

D0                 A0

ULONG AddClipCut(struct ClipAction *);

FUNCTION

Add another sub-clip definition to the currently open ClipCut list.

The fields in this structure give specifics for this sub-clip,

including:

Volume:Path/name for sub-clip

Beginning and ending field numbers

Contents (video and/or audio)

See StartClipCutList for a full description of this processing

mechanism.

INPUTS

subclip - a ClipAction structure specifying the new sub-clip.  The

same structure may be used for each call, as all needed

information is copied out of it before this function

returns.

RESULT

EXAMPLE

NOTES

The ClipAction fields AudStartField/VidStartField and

AudFieldCount/VidFieldCount must each match, regardless of the type

of clip specified to make.

BUGS

SEE ALSO

StartClipCutList

EndClipCutList

flyer.library/AddSeqClip                             flyer.library/AddSeqClip

NAME

AddSeqClip - Add an entry to the Flyer sequence

SYNOPSIS

error = AddSeqClip(clip)

D0                 A0

ULONG AddSeqClip(struct ClipAction *);

FUNCTION

Add another event for the Flyer's internal sequencer to play.

Any combination of Video and Audio in/out points is supported

properly, including split audio.

See NewSequence for more info about the Flyer's sequencer.

INPUTS

clip - a ClipAction structure specifying the event.  The same

structure may be used for each call, as all needed information

is copied out of it before this function returns.

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

EndSequence

EndSequenceNew

NewSequence

PlaySequence

flyer.library/AppendFields                         flyer.library/AppendFields

NAME

AppendFields - Capture live video field(s) and append to Flyer clip

SYNOPSIS

error = AppendFields(clip)

D0                   A0

ULONG AppendFields(struct ClipAction *);

FUNCTION

Captures live video field(s) and appends them to the specified Flyer

video clip.  Creates a new clip if it does not already exist.

Grabs correct field(s) from the captured color frames so that any

number of fields may be grabbed without concern for color phase.

Number of fields to record is specified in clip->ca_VidFieldCount.

This function always captures a new color frame.  Also, if the number

of fields specified spans more than one color frame, a new one is

captured for every new field 1 needed.  For example, if the current

clip needs a field 4 to be appended next and this function is called

with fields=3, a color frame is captured, and field 4 is appended.

Then a NEW color frame is captured and fields 1 and 2 are appended.

If this function is then called again with fields=3, a NEW color

frame is captured and fields 3 and 4 are appended.

INPUTS

clip - a ClipAction structure specifying the clip.

RESULT

EXAMPLE

NOTES

This function is not guaranteed to capture consecutive color frames,

as the processing delays incurred may prohibit this.  This may make

capturing more than 4 fields at a time somewhat useless, yet perhaps

interesting.

This function does not fully support the CAF_VIDEO and CAF_AUDIOL/R

flags.  It always captures video only without audio.  It is also not

currently capable of capturing audio only.

Be careful when appending fields onto a clip that was recorded

"live", as no checking is done to see that the attributes of appended

fields are correct for the rest of the clip (such as VIDEO and/or

AUDIO flags).  Since audio is not currently supported by this

function, just be sure that fields are appended only to video-only

clips (or just build new clips using this function).

BUGS

SEE ALSO

flyer.library/BeginFindField                     flyer.library/BeginFindField

NAME

BeginFindField - Prepare to shuttle/jog a clip

SYNOPSIS

error = BeginFindField(clipaction)

D0                     A0

ULONG BeginFindField(struct ClipAction *);

FUNCTION

Prepares Flyer for a shuttle/jog session for the named clip.  Call

this when the user brings up the control panel for a clip.

You must prepare a ClipAction structure with the desired parameters

then pass a pointer to it to this routine, which allows the Flyer to

prepare itself internally.  All calls to DoFindField, FindFieldAudio,

and EndFindField must be passed this same structure pointer.

See also FindFieldAudio.

The fields which need setup prior to calling BeginFindField:

ca_Volume       Ptr to FlyerVolume structure

ca_Channel      Video channel to use during session

ca_Flags        CAB_VIDEO to see found frames

CAB_AUDIO1 and/or 2 to hear found frames

(here it is legal to set none)

ca_VolSust1

ca_VolSust2     Volume for audio channels

fv_Path         Name of clip -- if volume name is prepended,

then the next 3 fields can be left blank

fv_Board        Flyer board number

fv_SCSIchannel  SCSI channel on which clip resides (optional)

fv_SCSIdrive    Drive on SCSI channel on which clip resides

(optional)

This call should always have a matching EndFindField call eventually.

Do not call this twice without an intervening call to EndFindField.

The result from this call should be checked.  A 0 value indicates all

went well, the Flyer is prepared for "DoFindField" calls.  Any non-0

value indicates a failure, most likely that the named file could not

be found on the specified drive (do not call EndFindField on it).

INPUTS

clipaction - specifies the name of the clip

RESULT

EXAMPLE

NOTES

This call does not return until ready for DoFindField calls (???)

BUGS

SEE ALSO

FindFieldAudio

flyer.library/ChangeAudio                           flyer.library/ChangeAudio

NAME

ChangeAudio - Change audio parameters of a clip that is in progress

SYNOPSIS

error = ChangeAudio(clipaction)

D0                  A0

ULONG ChangeAudio(struct ClipAction *);

FUNCTION

This routine allows a clip that is in progress to have its audio

parameters adjusted.  Simply modify the audio field(s) desired

(or the CAB_AUDIO1/2 flags) in the structure used to initially start

the clip, then call this function with a pointer to that structure.

INPUTS

clipaction - pointer to same structure clip was initiated with

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/CheckAction                           flyer.library/CheckAction

NAME

CheckAction - Check progress of an actions

SYNOPSIS

status = CheckAction(action)

D0                   A0

ULONG CheckAction(struct ClipAction *);

FUNCTION

Checks if the operation associated with the provided ClipAction

pointer has finished or not.  Returns FERR_OKAY if the action has

finished, or FERR_BUSY if it is still in progress.

Also, starting with rev 4.08, updates the (new) ca_Status field with

current status for the command.  This data is only available with

certain commands that use ClipAction structures.

INPUTS

action - pointer to structure that was used to issue the original

command

RESULT

status = FERR_OKAY or FERR_BUSY

EXAMPLE

NOTES

BUGS

SEE ALSO

WaitAction

flyer.library/CloseField                             flyer.library/CloseField

NAME

CloseField - Closes an OpenReadField or (Easy)OpenWriteField

SYNOPSIS

error = CloseField(action)

D0                 A0

ULONG CloseField(struct ClipAction *);

FUNCTION

Closes the field which was previously opened using an OpenReadField,

OpenWriteField, or EasyOpenWriteField call.  In the case of a write

session being closed, any unwritten data is written to the clip.

Also, if less than a full field of scan lines was written, fills in

remainder with fill color (usually black).

Returns FERR_FULL if not enough room left in current field to write

any remaining data.

The only structure field which needs setup prior to calling

CloseField:

ca_FldHandle   Field handle returned from successful

OpenReadField or (Easy)OpenWriteField call

INPUTS

action - pointer to structure which contains field handle to close

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

EasyOpenWriteField

OpenReadField

OpenWriteField

SetFillColor

flyer.library/CopyData                                 flyer.library/CopyData

NAME

CopyData - Copy data from one location to another

SYNOPSIS

error = CopyData(srcvolume,destvolume,srcaddr,blocks,destaddr)

D0               A0        A1         D0      D1     D2

ULONG CopyData(struct FlyerVolume *,struct FlyerVolume *,ULONG,ULONG,

ULONG);

FUNCTION

Copies a range of data from one drive to another.  This currently

works with a start block number and a block count.  The start

locations may be different on the src and dest drives.  This function

may also be used to move data on the same drive.  Handles making a

copy which overlaps original on same drive.

Can also read/write to/from a tape drive by simply using -1 for the

appropriate address (srcaddr or destaddr).

INPUTS

srcvolume - pointer to structure which describes a volume (used to

pick specific Flyer card).

destvolume - pointer to structure which describes a volume (used to

pick specific Flyer card).

srcaddr - SCSI block address on source drive

blocks - number of SCSI blocks to copy

destaddr - SCSI block address on destination drive

RESULT

EXAMPLE

NOTES

May copy slower than "real-time" playback rate if copying to and from

the same drive.

BUGS

SEE ALSO

flyer.library/CPUDMA                                     flyer.library/CPUDMA

NAME

CPUDMA -- Transfer data between DMA memory and CPU memory

SYNOPSIS

error = CPUDMA(board,cpuptr,dmaptr,length,readflag)

D0             D0    A0     A1     D1     D2

ULONG CPUDMA(UBYTE,ULONG,ULONG,UWORD,UBYTE);

FUNCTION

This function transfers blocks of data between the Flyer's DRAM and SRAM

areas.  All pointers and sizes are in blocks (512 bytes).  "Writes" are

TO SRAM, "reads" are FROM SRAM.

CAUTION! These memory areas are highly private and dangerous to access.

Use only under advisement or based on sample code.

INPUTS

board - specifies the Flyer board (0-3)

cpuptr - CPU address (block)

dmaptr - DMA address (block)

length - length of transfer (in blocks)

readflag - 0=write, 1=read

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/Defaults                                 flyer.library/Defaults

NAME

Defaults - clear given ClipAction structure(s) to default values

SYNOPSIS

error = Defaults(clipaction)

D0               A0

VOID Defaults(struct ClipAction *);

FUNCTION

Clears the given ClipAction structure to default values, as well as

the attached FlyerVolume structure.  See structure documentation for

default values.

INPUTS

clipaction - pointer to a ClipAction structure.  Will also setup

FlyerVolume structure, if attached.

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.h

flyer.library/DoFindField                           flyer.library/DoFindField

NAME

DoFindField - find a specific field in clip (and view/hear it)

SYNOPSIS

error = DoFindField(clipaction)

D0                  A0

ULONG DoFindField(struct ClipAction *);

FUNCTION

Finds the color frame that contains the field number specified in

ca_VidStartField.  If the CAB_VIDEO flag was set, the frame's video

will loop on the output channel.  Also, if the CAB_AUDIO1/2 flag(s)

were set, the frame's audio will be heard.

Currently, when the user stops in a particular spot, the color frame

loops repeatedly, but the audio (if on) is heard once per new frame

only.

If the return value is non-0, something went wrong (such as the

requested field number is out of range for the clip).

INPUTS

clipaction - same pointer as was used with BeginFindField

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/EasyOpenWriteField             flyer.library/EasyOpenWriteField

NAME

EasyOpenWriteField - Open a clip field for writing (easy version)

SYNOPSIS

error = EasyOpenWriteField(action,field,modes,quality)

D0                         A0     D0    D1    D2

ULONG EasyOpenWriteField(struct ClipAction *,ULONG,UBYTE,UBYTE);

FUNCTION

Provides an easier front-end for the more complicated OpenWriteField

call.

See the description under OpenWriteField for a full description of

field writing and the "action", "field", and "modes" arguments.

INPUTS

action - pointer to structure which describes a volume and the name

of the clip to operate on.

field - field number of clip (starts at 0).  Is a don't care with

some open modes

modes - flags describing how to handle writing field

quality - a number representing the video quality

Currently supported modes, in order of decreasing video quality:

0 (D2)  Best quality, worst compression

1 (D2)

2 (SN)

3 (SN)

4 (SN)  Worst quality, best compression

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

CloseField

OpenReadField

OpenWriteField

FlyerWriteLine

flyer.library/EndClipCutList                     flyer.library/EndClipCutList

NAME

EndClipCutList - Finalizes ClipCut list

SYNOPSIS

error = EndClipCutList(doit)

D0                     D0

ULONG EndClipCutList(UBYTE);

FUNCTION

Finalizes a ClipCut list that was opened with StartClipCutList.  If

the "doit" flag is set, the processing will begin.  Otherwise, the

list is thrown away and the original clip remains unchanged.

See StartClipCutList for a full description of this processing

mechanism.

INPUTS

doit - flag: 0 aborts and discards list, 1 starts clip processing

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

StartClipCutList

AddClipCut

flyer.library/EndFindField                         flyer.library/EndFindField

NAME

EndFindField - Cleanup after a shuttle/jog session

SYNOPSIS

error = EndFindField(clipaction)

D0                   A0

ULONG EndFindField(struct ClipAction *);

FUNCTION

This call frees up resources allocated with a BeginFindField call.

Call when the control panel for a clip is put away.  You must pass

a pointer to the same structure as was passed to BeginFindField.

If CAF_USEMATTE flag is true in the ClipAction structure, this call

will also put up the specified matte color on the video channel.

A return value of 0 indicates all went well.

INPUTS

clipaction - same pointer as was used for entire shuttle/jog session.

RESULT

EXAMPLE

NOTES

Only matte black is currently supported for CAF_USEMATTE

BUGS

SEE ALSO

flyer.library/EndHeadList                           flyer.library/EndHeadList

NAME

EndHeadList - Completes list of A/B heads

SYNOPSIS

error = EndHeadList(board,makeit)

D0                  D0    D1

ULONG EndHeadList(UBYTE,UBYTE);

FUNCTION

Completes list of A/B heads.  If 'makeit' is 0, the list is thrown

away (aborted).  Otherwise, the Flyer then begins creating heads.

It may use old clip heads that already exist or create new ones.

Any old heads that are not used in the list are deleted.

INPUTS

board - specifies the Flyer board (0-3)

makeit - flag

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/EndSequence                           flyer.library/EndSequence

NAME

EndSequence - Finalizes the Flyer's internal sequence

SYNOPSIS

error = EndSequence(board,doit)

D0                  D0    D1

ULONG EndSequence(UBYTE, UBYTE);

FUNCTION

Finalizes the sequence definition that was downloaded.  Post process-

ing occurs at this time, such as sequence optimization and temporary

data movement.  This call, therefore, may take a while to complete.

When it does, the sequence is ready to play (using PlaySequence).

If the "doit" flag is FALSE (0), no post processing is done, but the

sequence is closed (this is required as an "abort" during sequence

downloading).

See NewSequence for more info on Flyer sequencing.

INPUTS

board - specifies the Flyer board (0-3)

doit - flag: 0 aborts and discards sequence, 1 starts post-processing

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

AddSeqClip

EndSequenceNew

NewSequence

PlaySequence

flyer.library/EndSequenceNew                     flyer.library/EndSequenceNew

NAME

EndSequenceNew -- Finalizes the Flyer's internal sequence (extra features)

SYNOPSIS

error = EndSequenceNew(action, doit)

D0                     A0      D0

ULONG EndSequenceNew(struct ClipAction *, UBYTE);

FUNCTION

Identical to EndSequence function, except that it uses a ClipAction structur

e,

which specifies the Flyer board number.  This allows some enhanced things

during the sometimes lengthy sequence processing phase, such as the ability

to be run asynchronously, ability to be aborted, and the ability for the

application to obtain status during this phase.

INPUTS

clipaction - specifies the board number in the attached Volume structure

doit - flag: 0 aborts and discards sequence, 1 starts post-processing

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

AddSeqClip

EndSequence

NewSequence

PlaySequence

flyer.library/Error2String                         flyer.library/Error2String

NAME

Error2String - Convert a Flyer error code into an error string

SYNOPSIS

desc = Error2String(error)

D0

char * Error2String(UBYTE);

FUNCTION

Gives an descriptive string for the supplied Flyer error code.

Simply returns a pointer to the string.  DO NOT MODIFY THE DATA IN

THIS STRING.

INPUTS

error - an error code returned by a Flyer call

RESULT

desc - pointer to a static string which describes the error condition

EXAMPLE

NOTES

Does not currently convert some of the more "internal" Flyer errors,

but just gives "???".

BUGS

SEE ALSO

flyer.library/FindDrives                             flyer.library/FindDrives

NAME

FindDrives -- Find responding drives on SCSI bus

SYNOPSIS

error = FindDrives(flyervolume,buffer)

D0                 A0          A1

ULONG FindDrives(struct FlyerVolume *,APTR);

FUNCTION

This function scans one of the Flyer's SCSI busses, looking for

drives at each of the possible unit numbers.  An array of data

is returned which gives some rudimentary information about which

unit numbers correspond to a present drive, as well as some info

which is helpful in getting more detailed data (with the Inquiry

command.

INPUTS

volume - pointer to structure which specifies bus to scan for drives

buffer - pointer to an 18 byte buffer which receives results

RESULT

Format of data array:

UBYTE DriveFlags;  // '1' bit at (1<<unit) for each drive present

UBYTE pad;

UBYTE Versions[8]; // SCSI versions of each drive found, [x] = unit

UBYTE InqLens[8];  // Inquiry lengths of each drive found, [x] = unit

EXAMPLE

NOTES

BUGS

SEE ALSO

Inquiry

flyer.library/FindFieldAudio                     flyer.library/FindFieldAudio

NAME

FindFieldAudio - change audio parameters during shuttle/jog session

SYNOPSIS

error = FindFieldAudio(clipaction)

D0                     A0

ULONG FindFieldAudio(struct ClipAction *);

FUNCTION

This call allows you to change the status of the audio flag while in

the middle of a shuttle session (overrides the initial audioflag

specified in BeginFindField).  To effect the change, modify the

CAB_AUDIO flags in ca_Flags and pass the structure to this routine.

Call this as many times as needed (whenever the user clicks the audio

button on/off), but do not call it outside of the Begin/EndFindField

pair.

A return value of 0 indicates all went well.

INPUTS

clipaction - same pointer as was used for entire shuttle/jog session.

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/Firmware                                 flyer.library/Firmware

NAME

Firmware - Download and run software on Flyer CPU

SYNOPSIS

error = Firmware(board,length,data,offset)

D0               D0    D1     A0   D2

ULONG Firmware(UBYTE,ULONG,APTR,ULONG);

FUNCTION

Downloads the provided binary file to the Flyer and executes it

as the controlling software.

INPUTS

board - specifies the Flyer board (0-3)

length - length of data provided

offset - offset address in shared SRAM

data - pointer to binary data

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/FlyerAudioCtrl                     flyer.library/FlyerAudioCtrl

NAME

FlyerAudioCtrl - Sense/control audio rec level/aux input functions

SYNOPSIS

error = FlyerAudioCtrl(board,FlyAudCtrl,oper)

D0                     D0    A0         D1

ULONG FlyerAudioCtrl(UBYTE,struct FlyAudCtrl *,UBYTE);

FUNCTION

Provides access to the Flyer's audio subsystem.  This provides a

means of smartly setting the input gain on record, as well as control

over the Flyer's auxilliary audio inputs.

The "oper" flags describe which portions of the FlyAudCtrl structure

to apply.  This allows modification of individual values, and the

ability to sense input levels without changing any values.

INPUTS

board - specifies the Flyer board (0-3)

FlyAudCtrl - pointer to FlyAudCtrl structure

oper - various flags indicating what kind of operation(s) to perform:

FACOF_SENSE   -- update LeftSense/RightSense values

FACOF_SENSE8  -- update LeftSense/RightSense with 8-bit values

FACOF_SETGAIN -- set input gain (for recording)

FACOF_SETSRC  -- set the input selector mux

FACOF_SETMIX  -- set auxilliary channel mixing values

Any combination of these operations can be specified.

RESULT

EXAMPLE

NOTES

The LeftSense and RightSense values from FACOF_SENSE are interpreted:

0 -- over -1.0 dB underrange

1 -- 0 to -1.0 dB underrange

2 -- 0 to 1.0 dB overrange

3 -- over 1.0 dB overrange

FACOF_SENSE8 causes Left/RightSense to contain 8 bit peak-reading

values (low 8 bits truncated off)

BUGS

SEE ALSO

flyer.h

flyer.library/FlyerCopyClip                       flyer.library/FlyerCopyClip

NAME

FlyerCopyClip - Fast copy a flyer clip

SYNOPSIS

error = FlyerCopyClip(srcvolume,destvolume)

D0                    A0        A1

ULONG FlyerCopyClip(struct FlyerVolume *,struct FlyerVolume *);

FUNCTION

Makes a copy of a Flyer clip using high speed copying (independent of

Amiga host operating system).  Will fail if filename is not found

on the source volume or if the destination filename already exists on

the destination volume.  Will not create subdirectories for the

destination name, so ensure entire path exists before starting copy.

Source and destination volumes may be the same drive, but copying

will be slower.

INPUTS

srcvolume - pointer to structure which describes the source clip's

path/name and the volume on which it is found.

destvolume - pointer to structure which describes the destination

path/name and the volume on which to create it.  Must

always contain the path/name, even if not renaming clip

during copy.

RESULT

EXAMPLE

NOTES

Both source and destination volumes must be attached to the same

Flyer card.

BUGS

SEE ALSO

FlyerCopyClipNew

flyer.library/FlyerCopyClipNew                 flyer.library/FlyerCopyClipNew

NAME

FlyerCopyClipNew -- Fast copy a flyer clip (w/status & abort capabilities)

SYNOPSIS

error = FlyerCopyClipNew(srcaction,destvolume)

D0                       A0        A1

ULONG FlyerCopyClipNew(struct ClipAction *,struct FlyerVolume *);

FUNCTION

Identical to FlyerCopyClip function, except that it uses a ClipAction struct

ure

to specify the source, which adds the ability to run it asynchronously, abil

ity

to be aborted, and the ability to obtain status during a copy.

INPUTS

srcaction - pointer to structure which describes the source volume and clip

name.  "ReturnTime" and "Status" fields are also used.

destvolume - pointer to structure which describes the destination

path/name and the volume on which to create it.  Must

always contain the path/name, even if not renaming clip

during copy.

RESULT

EXAMPLE

NOTES

Both source and destination volumes must be attached to the same

Flyer card.

BUGS

SEE ALSO

FlyerCopyClip

flyer.library/FlyerCreateDir                     flyer.library/FlyerCreateDir

NAME

FlyerCreateDir -- create a sub-directory on a Flyer drive

SYNOPSIS

error = FlyerCreateDir(clipaction)

D0                     A0

ULONG FlyerCreateDir(struct ClipAction *);

FUNCTION

INPUTS

clipaction - specifies volume/path/name of the directory to create

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/FlyerDeFrag                           flyer.library/FlyerDeFrag

NAME

FlyerDeFrag - De-fragment hard drive

SYNOPSIS

error = FlyerDeFrag(volume)

D0                  A0

ULONG FlyerDeFrag(struct FlyerVolume *);

FUNCTION

Begins defragmentation process on specified drive.

INPUTS

volume - pointer to structure which describes the volume to defrag

RESULT

EXAMPLE

NOTES

Currently accepts no parameters and cannot be interrupted

BUGS

Reports of bugs.  Unable to reproduce to date...

SEE ALSO

FlyerDeFragNew

flyer.library/FlyerDeFragNew                     flyer.library/FlyerDeFragNew

NAME

FlyerDeFragNew -- De-fragment hard drive (extra features)

SYNOPSIS

error = FlyerDeFragNew(clipaction)

D0                     A0

ULONG FlyerDeFragNew(struct ClipAction *);

FUNCTION

Begins defragmentation process on specified drive.  This is identical to

FlyerDeFrag function, except that this one uses a ClipAction structure

to specifies the Flyer drive.  This allows some enhanced things during

defragmentation, such as the ability to be run asynchronously, ability

to be aborted, and the ability for the application to obtain status

while it's occurring.

INPUTS

clipaction - specifies the drive using an attached FlyerVolume structure

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

FlyerDeFrag

flyer.library/FlyerDelete                           flyer.library/FlyerDelete

NAME

FlyerDelete -- Delete a file from a Flyer drive

SYNOPSIS

error = FlyerDelete(clipaction)

D0                  A0

ULONG FlyerDelete(struct ClipAction *);

FUNCTION

INPUTS

clipaction - specifies the path/name of the file to delete

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/FlyerDirList                         flyer.library/FlyerDirList

NAME

FlyerDirList -- return first/next entry in a directory

SYNOPSIS

error = FlyerDirList(flyervolume,grip,objinfoptr,firstflag,fsonly)

D0                   A0          D0   A1         D1        D2

ULONG FlyerDirList(struct FlyerVolume *,ULONG,struct ClipInfo *,

UBYTE,UBYTE);

FUNCTION

INPUTS

volume - pointer to structure which specifies drive

grip - grip of directory

objinfoptr - Pointer to ClipInfo structure to receive info

firstflag - 0 if first call, 1 for each additional

fsonly - 0 for full information, 1 for just FileSys info

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/FlyerDriveCheck                   flyer.library/FlyerDriveCheck

NAME

FlyerDriveCheck - check if the specified drive has anything in it

SYNOPSIS

error = FlyerDriveCheck(volume)

D0                      A0

ULONG FlyerDriveCheck(struct FlyerVolume *);

FUNCTION

Checks to see if the specified board/channel/drive has media loaded.

Note that you must use the FVF_USENUMS flags, since the use of a

volume name is not logical here.

INPUTS

volume - pointer to structure which describes a volume

RESULT

error - FERR_OKAY or FERR_VOLNOTFOUND

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/FlyerDriveInfo                     flyer.library/FlyerDriveInfo

NAME

FlyerDriveInfo - Return general information about a drive

SYNOPSIS

error = FlyerDriveInfo(volume,volinfo)

D0                     A0     A1

ULONG FlyerDriveInfo(struct FlyerVolume *,struct FlyerVolInfo *);

FUNCTION

This returns general information about the drive, including the

volume name, total number of blocks, number of blocks free,

size of largest contiguous block, and free block size if DeFrag

would be performed.

If volptr is NULL, just fills in info in FlyerVolume structure only.

INPUTS

volume - pointer to structure which specifies volume

volinfo - pointer to structure to receive information about volume

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/FlyerFileClose                     flyer.library/FlyerFileClose

NAME

FlyerFileClose -- close a file

SYNOPSIS

error = FlyerFileClose(flyervolume,fileID)

D0                     A0          D0

ULONG FlyerFileClose(struct FlyerVolume *,ULONG);

FUNCTION

INPUTS

volume - pointer to same structure as passed to FlyerFileOpen

fileID - ID returned from FlyerFileOpen call

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

FlyerFileOpen

flyer.library/FlyerFileOpen                       flyer.library/FlyerFileOpen

NAME

FlyerFileOpen -- open a file on a Flyer drive for reading/writing

SYNOPSIS

error = FlyerFileOpen(clipaction)

D0                    A0

ULONG FlyerFileOpen(struct ClipAction *);

FUNCTION

INPUTS

clipaction - specifies the name of the file to open

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/FlyerFileRead                       flyer.library/FlyerFileRead

NAME

FlyerFileRead -- read from an open Flyer file

SYNOPSIS

error = FlyerFileRead(flyervolume,fileID,size,buffer,actual)

D0                    A0          D0     D1   A1     A2

ULONG FlyerFileRead(struct FlyerVolume *,ULONG,ULONG,UBYTE *,

ULONG *);

FUNCTION

INPUTS

volume - pointer to same structure as passed to FlyerFileOpen

fileID - ID returned from FlyerFileOpen call

size - number of bytes to read

buffer - pointer to buffer to receive data

actual - pointer to variable to receive count of actual bytes read

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/FlyerFileSeek                       flyer.library/FlyerFileSeek

NAME

FlyerFileSeek -- seek to a given position in a file

SYNOPSIS

error = FlyerFileSeek(flyervolume,fileID,pos,mode,posptr,oldposptr)

D0                    A0          D0     D1  D2   A1     A2

ULONG FlyerFileSeek(struct FlyerVolume *,ULONG,LONG,UBYTE,LONG *,

LONG *);

FUNCTION

INPUTS

volume - pointer to same structure as passed to FlyerFileOpen

fileID - ID returned from FlyerFileOpen call

pos - requested new file position (per 'mode' below)

mode - seek mode to apply to 'pos' (FLYER_POS_xxx)

posptr - pointer to variable to receive new file position

oldposptr - pointer to variable to receive old file position

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.h

flyer.library/FlyerFileWrite                     flyer.library/FlyerFileWrite

NAME

FlyerFileWrite -- write to an open Flyer file

SYNOPSIS

error = FlyerFileWrite(flyervolume,fileID,size,buffer,actual)

D0                     A0          D0     D1   A1     A2

ULONG FlyerFileWrite(struct FlyerVolume *,ULONG,ULONG,UBYTE *,

ULONG *);

FUNCTION

INPUTS

volume - pointer to same structure as passed to FlyerFileOpen

fileID - ID returned from FlyerFileOpen call

size - number of bytes to write

buffer - pointer to buffer which contains data

actual - pointer to variable to receive count of actual bytes written

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/FlyerFormat                           flyer.library/FlyerFormat

NAME

FlyerFormat - High-level format a drive with the Flyer's filesystem

SYNOPSIS

error = FlyerFormat(volume,name,datestamp,blocks,flags)

D0                  A0     A1   A2        D0     D1

ULONG FlyerFormat(struct FlyerVolume *,char *,struct DateStamp *,

ULONG,UBYTE);

FUNCTION

This function does a high-level format on a drive connected to the

Flyer.  Not all sectors are read/write tested, so this is a "quick"

format.  The format procedure normally uses the entire drive, but

this can be reduced to avoid using slower parts of the drive.

INPUTS

volume - NULL string, specifies drive to format

name - pointer to a null-terminated string to use for the volume name

datestamp - pointer to an AmigaDOS DateStamp structure to use as the

drive's creation date

blocks - NULL for entire drive, or the number of sectors to use for

video data.  WARNING!  This does not actually prohibit the

Flyer from using the remaining space, but gives a cutoff

point beyond which no video clips may be placed.  Non time-

critical data may eventually be placed in this "slow" region.

flags - FVIF_xxx flags to request to be applied to this drive

(specifically FVIF_VIDEOREADY and FVIF_AUDIOREADY).  FlyerFormat

will test the speed of the drive and clear the video flag if it

does not find it capable.  This allows drives to be targeted

as data only, data/audio, or data/audio/video.

RESULT

EXAMPLE

NOTES

Do not use an fv_Path string to specify the drive.  Specify a NULL

string and specify the exact drive specifically with fv_SCSIdrive.

This will prevent formatting the wrong drive if two exist with

identical volume names!

BUGS

SEE ALSO

flyer.h

flyer.library/FlyerInputSel                       flyer.library/FlyerInputSel

NAME

FlyerInputSel - Select Flyer video input sources

SYNOPSIS

error = FlyerInputSel(board,video,sync)

D0                    D0    D1    D2

ULONG FlyerInputSel(UBYTE,UBYTE,UBYTE);

FUNCTION

Specifies what video channel to use for recording to the Flyer and

where to get sync.

INPUTS

board - specifies the Flyer board (0-3)

video - video source to record

FI_Camcorder   = Flyer camcorder input (TBC required)

FI_SVHS        = Flyer SVHS input (TBC required)

FI_Toaster1    = Toaster input 1

FI_Toaster2    = Toaster input 2

FI_ToasterMain = Toaster Main bus output

FI_ToasterPV   = Toaster Preview bus output

sync - video source to use as a reference

FS_ToasterMain = Toaster Main output

FS_Toaster1    = Toaster input 1

RESULT

EXAMPLE

NOTES

Any changes to video or sync source using this command should be

given sufficient time to "settle" before beginning to record.

BUGS

SEE ALSO

FlyerTermination

ToasterMux

flyer.library/FlyerPlay                               flyer.library/FlyerPlay

NAME

FlyerPlay - Play a video/audio clip

SYNOPSIS

error = FlyerPlay(clipaction)

D0                A0

ULONG FlyerPlay(struct ClipAction *);

FUNCTION

Plays a video/audio clip as specified in the structure whose pointer

is given.  The definition of this structure is in "Flyer.h".

This call can be made to return at different times, depending on the

value of ReturnTime.

If the video channel or SCSI channel needed to accomplish this action

are in use when this clip needs to begin, the PermissFlags indicate

what actions the Flyer can take to free up the necessary resource(s).

CAPB_STEALOURVIDEO allows the Flyer to stop a clip on the video

channel specified for this new play.  CAPB_KILLOTHERVIDEO allows the

Flyer to stop clips on other video channels if needed to gain access

to the SCSI drive for this new clip.

If the CAPB_ERRIFBUSY flag is set, this call will return with error

FERR_CHANINUSE if the clip cannot be played without waiting for other

resources.  If this flag is not set, the Flyer will delay playback if

needed while waiting for resources it needs.

When using a ReturnTime of RTT_STOPPED, you may modify/recycle the

ClipAction structure once this call returns.  For RTT_IMMED and

RTT_STARTED, you must not modify the ClipAction structure until the

clip stops or is aborted.  Use AbortAction() to abort playback, and

CheckProgress(), CheckAction(), or WaitAction() to determine when

it's safe to reuse the structure.

If CAF_USEMATTE flag is true in the ClipAction structure, the video

channel this function uses will change to the matte color specified

in MatteY,MatteI,MatteQ fields when the clip finishes or is stopped.

INPUTS

clipaction - pointer to structure containing all information needed

for playback, and to receive results when done.

RESULT

EXAMPLE

NOTES

Only matte black is currently supported for CAF_USEMATTE

BUGS

SEE ALSO

flyer.library/FlyerQuit                               flyer.library/FlyerQuit

NAME

FlyerQuit -- Stop Flyer execution, return to boot ROM

SYNOPSIS

error = FlyerQuit(board)

D0           D0

ULONG FlyerQuit(UBYTE);

FUNCTION

INPUTS

board - specifies the Flyer board (0-3)

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/FlyerReadCalib                     flyer.library/FlyerReadCalib

NAME

FlyerReadCalib - Inspect the Flyer's calibration registers

SYNOPSIS

error = FlyerReadCalib(board,item,valueptr)

D0                     D0    D1   A0

ULONG FlyerReadCalib(UBYTE,UWORD,WORD *);

FUNCTION

Reads the specified Flyer calibration register.  See flyer.h for

the "item" values (CALIB_xxxx).  Value placed at pointer "valueptr".

INPUTS

board - specifies the Flyer board (0-3)

item - which register to change (see flyer.h)

valueptr - pointer to a UWORD to fill in with the value

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

FlyerWriteCalib

flyer.library/FlyerReadLine                       flyer.library/FlyerReadLine

NAME

FlyerReadLine - Read a scan line from a field previously opened

SYNOPSIS

error = FlyerReadLine(action,buffer)

D0                    A0     A1

ULONG FlyerReadLine(struct ClipAction *,UBYTE *);

FUNCTION

Decompresses next scan line from open field and transfers into

caller's buffer (must be big enough to receive 752 bytes).  NTSC line

21 is the first line read from the field, and 262 is the last.  Any

extra calls will fill the buffer with the fill color (usually black).

This function does software emulation of the Flyer's hardware which

converts VTASC-compressed data into D2 data, including FIR filtering.

The fields which need setup prior to calling FlyerReadLine:

ca_FldHandle  - Field handle from successful OpenReadField

ca_ReturnTime - RT_xxx value desired (not currently supported)

INPUTS

action - pointer to structure which contains the field handle to read

from and the return time for this call.

buffer - buffer to receive composite scan line data

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

CloseField

OpenReadField

SetFillColor

FlyerWriteLine

flyer.library/FlyerRecord                           flyer.library/FlyerRecord

NAME

FlyerRecord - Record a video/audio clip

SYNOPSIS

error = FlyerRecord(clipaction)

D0                  A0

ULONG FlyerRecord(struct ClipAction *);

FUNCTION

Records a video/audio clip as specified in the structure whose

pointer is given.  The definition of this structure is in "Flyer.h".

Except when ReturnTime = RTT_STOPPED, recording can be stopped at any

time with the AbortAction() command (see below).

Do not call with both CAB_AUDIO1/2 and CAB_VIDEO flags clear, as this

is nonsensical.

If the video channel or SCSI channel needed to accomplish this action

are in use when this clip needs to begin, the PermissFlags indicate

what actions the Flyer can take to free up the necessary resource(s).

CAPB_STEALOURVIDEO allows the Flyer to stop a clip on the video

channel specified for this new record.  CAPB_KILLOTHERVIDEO allows

the Flyer to stop clips on other video channels if needed to gain

access to the SCSI drive for this new clip.

If the CAPB_ERRIFBUSY flag is set, this call will return with error

FERR_CHANINUSE if the clip cannot be recorded without waiting for

other resources.  If this flag is not set, the Flyer will delay

recording if needed while waiting for resources it needs.

When using a ReturnTime of RTT_STOPPED, you may modify/recycle the

ClipAction structure once this call returns.  For RTT_IMMED and

RTT_STARTED, you must not modify the ClipAction structure until the

clip stops or is aborted.  Use AbortAction() to abort recording, and

CheckProgress(), CheckAction(), or WaitAction() to determine when

it's safe to reuse the structure.

INPUTS

clipaction - pointer to structure containing all information needed

for recording, and to receive results when done.

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/FlyerRename                           flyer.library/FlyerRename

NAME

FlyerRename -- rename a file/dir on a Flyer drive

SYNOPSIS

error = FlyerRename(oldclip,newgrip,newname)

D0                  A0      D0      A1

ULONG FlyerRename(struct ClipAction *,ULONG,char *);

FUNCTION

INPUTS

oldclip - specifies the path/name of the file to rename

newgrip - base grip to which 'newname' is relative

newname - new path/name for file (relative to 'newgrip')

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/FlyerRenameDisk                   flyer.library/FlyerRenameDisk

NAME

FlyerRenameDisk -- rename a Flyer drive volume

SYNOPSIS

error = FlyerRenameDisk(flyervolume,newname)

D0                      A0          A1

ULONG FlyerRenameDisk(struct FlyerVolume *,char *);

FUNCTION

INPUTS

volume - pointer to structure which specifies drive

newname - pointer to new name string for volume

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/FlyerRunning                         flyer.library/FlyerRunning

NAME

FlyerRunning -- test if Flyer firmware is downloaded and running

SYNOPSIS

error = FlyerRunning(board)

D0                   D0

ULONG FlyerRunning(UBYTE);

FUNCTION

INPUTS

board - specifies the Flyer board (0-3)

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/FlyerSetBits                         flyer.library/FlyerSetBits

NAME

FlyerSetBits -- set protect bits for dir/file

SYNOPSIS

error = FlyerSetBits(flyervolume,grip,bits)

D0                   A0          D0   D1

ULONG FlyerSetBits(struct FlyerVolume *,ULONG,ULONG);

FUNCTION

INPUTS

volume - pointer to structure which specifies drive/path/name of file

grip - grip of file/dir

bits - new bits (32)

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/FlyerSetComment                   flyer.library/FlyerSetComment

NAME

FlyerSetComment -- set comment for dir/file

SYNOPSIS

error = FlyerSetComment(flyervolume,grip,comment)

D0                      A0          D0   A1

ULONG FlyerSetComment(struct FlyerVolume *,ULONG,char *);

FUNCTION

INPUTS

volume - pointer to structure which specifies drive/path/name of file

grip - grip of file/dir

comment - new comment string

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/FlyerSetDate                         flyer.library/FlyerSetDate

NAME

FlyerSetDate -- set date for file/dir

SYNOPSIS

error = FlyerSetDate(flyervolume,grip,days,minutes,ticks)

D0                   A0          D0   D1   D2      D3

ULONG FlyerSetDate(struct FlyerVolume *,ULONG,ULONG,ULONG,ULONG);

FUNCTION

INPUTS

volume - pointer to structure which specifies drive/path/name of file

grip - grip of file/dir

days - date: days

minutes - date: minutes

ticks - date: ticks

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/FlyerStripAudio                   flyer.library/FlyerStripAudio

NAME

FlyerStripAudio - Strip audio from a clip, make an audio-only clip

SYNOPSIS

error = FlyerStripAudio(srcvolume,destvolume)

D0                      A0        A1

ULONG FlyerStripAudio(struct FlyerVolume *,struct FlyerVolume *);

FUNCTION

Creates a new clip containing only the audio from the source clip.

Will fail if the source clip is not found or does not contain audio.

Destination clip must not already exist on the destination volume, or

an error will result.

Both source and destination volumes must be attached to the same

Flyer card.

INPUTS

srcvolume - pointer to structure which describes the source clip name

and the volume on which it is found.

destvolume - pointer to structure which describes the destination

clip name and the volume on which to place it.

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/FlyerTermination                 flyer.library/FlyerTermination

NAME

FlyerTermination - Set Flyer's video termination on/off

SYNOPSIS

error = FlyerTermination(board,flags)

D0                       D0    D1

ULONG FlyerTermination(UBYTE,UBYTE);

FUNCTION

Specifies which of the Flyer's 4 video terminators to turn on.

INPUTS

board - specifies the Flyer board (0-3)

flags - One flag for each of 4 terminators (0=off, 1=on)

Bit 0 = Toaster Input 1 terminator

Bit 1 = Toaster Input 3 terminator

Bit 2 = Toaster Input 4 terminator

Bit 3 = Toaster Main terminator

Power-up default is Inputs 3 & 4 terminated, Main and Input 1 not

terminated.

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

FlyerInputSel

ToasterMux

flyer.library/FlyerWriteCalib                   flyer.library/FlyerWriteCalib

NAME

FlyerWriteCalib - Manually set Flyer's calibration registers

SYNOPSIS

error = FlyerWriteCalib(board,item,value,saveflag)

D0                      D0    D1   D2    D3

ULONG FlyerWriteCalib(UBYTE,UWORD,WORD,UBYTE);

FUNCTION

Sets the value of one of the calibration registers.  See flyer.h for

the "item" values (CALIB_xxxx).  Starting with the Rev 4 board, these

values are kept in non-volatile memory on-board the Flyer.  To also

save the specified value to memory, set the "saveflag" argument.

INPUTS

board - specifies the Flyer board (0-3)

item - which register to change (see flyer.h)

value - item-specific value

saveflag - 0=just use value, 1=also save to non-volatile memory

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

FlyerReadCalib

flyer.library/FlyerWriteLine                     flyer.library/FlyerWriteLine

NAME

FlyerWriteLine - Write a scan line to a field previously opened

SYNOPSIS

error = FlyerWriteLine(action,buffer)

D0                     A0     A1

ULONG FlyerWriteLine(struct ClipAction *,UBYTE *);

FUNCTION

Transfers scan line data from caller's buffer, compresses it, and

places in the open clip as specified in (Easy)OpenWriteField call.

Expects 752 bytes from caller's buffer.  NTSC line 21 is expected on

the first call to this function, and 262 on the last.  Any extra

calls will be ignored.

This function does software emulation of the Flyer's hardware which

converts D2 data into VTASC-compressed data, including FIR filtering.

The fields which need setup prior to calling FlyerWriteLine:

ca_FldHandle   - Field handle from successful OpenWriteField

or EasyOpenWriteField.

ca_ReturnTime  - RT_xxx value desired (not currently supported)

Returns FERR_FULL if out of room in current field

INPUTS

action - pointer to structure which contains the field handle to

write to and the return time for this call.

buffer - contains composite scan line data (will be modified)

RESULT

EXAMPLE

NOTES

Currently modifies the data at the "buffer" pointer

BUGS

SEE ALSO

CloseField

EasyOpenWriteField

OpenWriteField

FlyerReadLine

flyer.library/GetClipInfo                           flyer.library/GetClipInfo

NAME

GetClipInfo - get information about a specific clip

SYNOPSIS

error = GetClipInfo(volume,clipinfo)

D0                  A0     A1

ULONG GetClipInfo(struct FlyerVolume *,struct ClipInfo *);

FUNCTION

Fills in the provided ClipInfo structure with information about the

specified clip.  Of particuar interest are: ci_fields equals the

number of fields the clip contains, ci_flags describes the type of

data in the clip.  See OIB_HASVIDEO and OIB_HASAUDIO in "Flyer.h".

Data files (such as icons) will have neither flag set.

You MUST initialize ci_len to the value CI_sizeof before calling this

function.  This is to ensure that future Flyer software does not

break old application software.

A non-zero return code indicates a failure (structure is not filled

in).  This call does not return until complete.  The structure may be

modified or reused in any way after it returns.

This routine is useful for obtaining info about a clip which has no

icon.

Also retrieves any starting SMPTE time-code from the clip, which can

be read using GetSMPTE.

INPUTS

volume - pointer to a FlyerVolume structure which describes the clip

clipinfo - pointer to structure to contain clip information

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

GetSMPTE

flyer.library/GetFieldClock                       flyer.library/GetFieldClock

NAME

GetFieldClock - Retrieve the Flyer's field counter

SYNOPSIS

error = GetFieldClock(clockptr)

D0                    A0

ULONG GetFieldClock(ULONG *);

FUNCTION

This returns the Flyer's internal field counter by plugging it into

the provided pointer to a ULONG.  If more than one Flyer exists, they

are automatically sync'd together by the flyer.library.  Therefore,

no board number or volume name is required for this function.

INPUTS

clockptr - pointer to ULONG to receive the clock value

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/GetFrameHeader                     flyer.library/GetFrameHeader

NAME

GetFrameHeader -- Read Frame Header structure from clip

SYNOPSIS

error = GetFrameHeader(action,buffer)

D0                     A0     A1

ULONG GetFrameHeader(struct ClipAction *,APTR);

FUNCTION

Retrieves a copy of a specific FrameHeader structure from an audio

or video clip.  FrameHeader chosen is the one that contains the field

number specified in ca_VidStartField (even for audio clips).

Places data at the structure pointed to by "buffer".

If the return value is not FERR_OKAY, something went wrong (such as

the clip was not found, or the requested field number is out of range).

Note: on success, clipaction->ca_StartBlk will contain the actual

block number where the frame header is found, and

clipaction->ca_Volume->fv_SCSIdrive will contain the actual drive number.

INPUTS

clipaction - specifies the volume/clip name and the desired field number

buffer - Pointer to caller's structure to fill in

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

PutFrameHeader

flyer.library/GetSMPTE                                 flyer.library/GetSMPTE

NAME

GetSMPTE - Return SMPTE time code information

SYNOPSIS

error = GetSMPTE(board,SMPTEinfo)

D0               D0    A0

ULONG GetSMPTE(UBYTE,struct SMPTEinfo *);

FUNCTION

This returns the last SMPTE time code information retrieved from a

clip.

This is generally used after a "DoFindField" call to retrieve the

SMPTE information related to that field, or after a GetClipInfo call

to get the start SMPTE time for the clip.

INPUTS

board - specifies the Flyer board (0-3)

SMPTEinfo - pointer to SMPTEinfo structure to receive time code info

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/InitFlyers                             flyer.library/InitFlyers

NAME

InitFlyers - setup all attached Flyer cards

SYNOPSIS

error = InitFlyers(lock)

D0                 D0

ULONG InitFlyers(BPTR);

FUNCTION

Perform setup on all Flyer boards present (programs all chips,

places all video channels in play mode, playing black).  Must be

called from a process so that library can access DOS functions.

Looks for all chip files needed in the directory which 'lock' is on.

INPUTS

lock - lock on directory in which to look for chip files

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/Inquiry                                   flyer.library/Inquiry

NAME

Inquiry -- Do SCSI Inquiry command

SYNOPSIS

error = Inquiry(flyervolume,buffersize,buffer)

D0              A0          D0         A1

ULONG Inquiry(struct FlyerVolume *,UBYTE,APTR);

FUNCTION

INPUTS

volume - pointer to structure which specifies drive

buffersize - size of buffer provided (in bytes)

buffer - pointer to buffer to receive Inquiry data

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/LocateField                           flyer.library/LocateField

NAME

LocateField - find a specific field in clip

SYNOPSIS

error = LocateField(clipaction)

D0                  A0

ULONG LocateField(struct ClipAction *);

FUNCTION

Finds the color frame that contains the field number specified in

ca_VidStartField.  This differs from the Begin/Do/EndFindField calls

in that this function just locates the field -- it does not attempt

to play its video or audio.  Also, the Begin/Do/End trio are designed

for multiple calls on the same clip (such as for jog/shuttling),

whereas this function is much simpler for just one lookup operation.

If the return value is not FERR_OKAY, something went wrong (such as

the requested field number is out of range for the clip).

INPUTS

clipaction - structure that contains the following data

ca_Volume         -- ptr to FlyerVolume structure which contains

the board, SCSI drive, and pathname for clip

ca_VidStartField  -- field number to locate

RESULT

If ERR_OKAY returned, clipaction->ca_StartBlk will contain the

block number of the frame header for the color frame which

contains the requested field

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/LockFlyVolList                     flyer.library/LockFlyVolList

NAME

LockFlyVolList - obtain lock on internal Flyer volumes list

SYNOPSIS

ptr = LockFlyVolList()

D0

struct MinList *LockFlyVolList(void);

FUNCTION

Returns a pointer to a MinList containing the currently mounted Flyer

volumes.  Also locks this list so that you may safely inspect it.  No

modifications to the list are allowed.  Be sure to release lock using

UnLockFlyVolList.

A return value of 0 indicates a failure.

INPUTS

RESULT

ptr - pointer to a MinList of Flyer Volume Node structures

(or 0 for failure)

EXAMPLE

NOTES

BUGS

SEE ALSO

UnLockFlyVolList

flyer.library/MakeClipHead                         flyer.library/MakeClipHead

NAME

MakeClipHead - Define an A/B head for the specified clip

SYNOPSIS

error = MakeClipHead(clipaction)

D0                   A0

ULONG MakeClipHead(struct ClipAction *);

FUNCTION

Define an A/B head for the specified clip.  Use the ca_VidStartField,

ca_AudStartField, ca_VidFieldCount, and ca_AudFieldCount entries to

specify where the head should start and how long it should be.

This function can be used in two ways.  If used by itself, the A/B

head is made immediately.  If used between StartHeadList and

EndHeadList calls, the definition is just added to an internal list

which will be created when EndHeadList is called with makeit=1.  The

second method can optimize your A/B heads and can take advantage of

heads already in existence to shorten its work load.  The immediate

method of MakeClipHead cannot do any of these optimizations.

INPUTS

clipaction - specifies the clip and the in/out points

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

VoidAllHeads

VoidCardHeads

VoidClipHead

flyer.library/MakeFlyerFile                       flyer.library/MakeFlyerFile

NAME

MakeFlyerFile - Create an empty file on a Flyer drive

SYNOPSIS

error = MakeFlyerFile(volume,blocks,startptr)

D0                    A0     D0     A1

ULONG MakeFlyerFile(struct FlyerVolume *,ULONG,ULONG *);

FUNCTION

Creates a file of a specified size on a Flyer drive and adds it to

the drive's filesystem.  The start block for the file data area is

returned to the caller, who may then fill the file with something

useful.

Previous library versions would only create files of a size which was

a multiple of 512.  Starting rev 4.04, MakeFlyerFile can be used to

create a file of any size, but for backward compatibility, here's how

you must specify the size:

Write the byte size into the variable that 'startptr' points to

Pass 'blocks' value of 0

Call MakeFlyerFile()

The value pointed to by 'startptr' has the same meaning on return as

before.

INPUTS

volume - pointer to structure which describes a volume and name for

new file

blocks - size of file in blocks (512 bytes each)

startptr - ptr to a ULONG to receive the start block reserved for

file's data.  This ULONG also contains the byte size of the file

to create, providing that 'blocks' is 0.

RESULT

EXAMPLE

NOTES

Errors will be returned if not enough contiguous space for the file,

or if a file of the same name already exists on that drive/path.

BUGS

SEE ALSO

FlyerCopyClip, FlyerCopyClipNew

flyer.library/ModeSelect                             flyer.library/ModeSelect

NAME

ModeSelect -- Do SCSI ModeSelect command

SYNOPSIS

error = ModeSelect(flyervolume,buffersize,buffer,PFbyte)

D0                 A0          D0         A1     D1

ULONG ModeSelect(struct FlyerVolume *,UBYTE,APTR,UBYTE);

FUNCTION

INPUTS

volume - pointer to structure which specifies drive

buffersize - size of buffer provided (in bytes)

buffer - pointer to buffer which contains ModeSelect data

PFbyte - SCSI PageFormat byte

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/ModeSense                               flyer.library/ModeSense

NAME

ModeSense -- Do SCSI ModeSense command

SYNOPSIS

error = ModeSense(flyervolume,buffersize,page,buffer)

D0                A0          D0         D1   A1

ULONG ModeSense(struct FlyerVolume *,UBYTE,UBYTE,APTR);

FUNCTION

INPUTS

volume - pointer to structure which specifies drive

buffersize - size of buffer provided (in bytes)

page - Mode page code to read

buffer - pointer to buffer to receive ModeSense data

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/NewSequence                           flyer.library/NewSequence

NAME

NewSequence - Prepare Flyer for a sequence download

SYNOPSIS

error = NewSequence(board)

D0                  D0

ULONG NewSequence(UBYTE);

FUNCTION

Used to begin sending a sequence definition to the Flyer.  Then,

using other calls, each piece of the sequence is defined, the

sequence is "closed", and then it may be played with one call.  This

is allows the Flyer to do much more complicated sequences

successfully than by using FlyerPlay calls in a double-buffered

fashion (which is now only supported in a limited way).

INPUTS

board - specifies the Flyer board (0-3)

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

AddSeqClip

EndSequence

EndSequenceNew

PlaySequence

flyer.library/OpenReadField                       flyer.library/OpenReadField

NAME

OpenReadField - Open a field from a clip for reading

SYNOPSIS

error = OpenReadField(action,field,modes)

D0                    A0     D0    D1

ULONG OpenReadField(struct ClipAction *,ULONG,UBYTE);

FUNCTION

Locates specified field of named clip and prepares to decompress

and transfer each scan line of the field using the FlyerReadLine

call.

This function, if successful, places a valid ca_FldHandle in the

ClipAction structure provided.  This same structure must be used for

any other calls relating to this open field, or you must manually

copy the value in ca_FldHandle into the ClipAction structure you wish

to use.

No compression information is required, as this information is

embedded in the clips themselves.

INPUTS

action - pointer to structure which describes a volume and the name

of the clip to operate on.

field - field number of clip (starts at 0)

modes - various flags

FRF_HALFLINES - allows reading the half lines.  Without this

flag set, half lines are skipped

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

CloseField

EasyOpenWriteField

OpenWriteField

FlyerReadLine

flyer.library/OpenWriteField                     flyer.library/OpenWriteField

NAME

OpenWriteField - Open a field from a clip for writing

SYNOPSIS

error = OpenWriteField(action,field,modes,compinfo)

D0                     A0     D0    D1    A1

ULONG OpenWriteField(struct ClipAction *,ULONG,UBYTE,

struct VidCompInfo *);

FUNCTION

Prepares to transfer and compress each scan line of a field using the

FlyerWriteLine call.  How the new data is integrated into the clip

depends on the "modes" flags specified:

FWF_NEW (field = dont care)

Writes the first field of a new clip (deletes old if exists)

FWF_APPEND (field = dont care)

Appends another field onto the clip

FWF_REWRITE (field = n)

Overwrites an existing field in the clip (must be same size or

smaller)

FWF_APPEND + FWF_REWRITE (field = dont care)

Replaces the last field.  Used for retrying with different

compression

FWF_APPEND + FWF_REWRITE + FWF_FRAME (field = n)

Rewrite multiple fields in the last color frame (each field

sequentially).  Field must be in the last color frame.  Used

for retrying entire color frame with different compression.

FWF_REWRITE + FWF_FRAME (field = n)

Rewrite multiple fields in a color frame (each field

sequentially).  Used for retrying entire color frame with

different compression.

FWF_HALFLINES allows writing of half lines.  Without this flag set,

half lines are skipped and padded.

"compinfo" points to a structure containing information about how to

compress the data.  If this pointer is NULL, the Flyer will default

to its best algorithm.

This function, if successful, places a valid ca_FldHandle in the

ClipAction structure provided.  This same structure must be used for

any other calls relating to this open field, or you must manually

copy the value in ca_FldHandle into the ClipAction structure you wish

to use.

This function may fail and return FERR_FULL if not enough contiguous

storage exists at the end of the clip to handle appending a field.

Also, field writing may fail if the data produced is too large for

the hardware to play.  An FERR_FULL error from FlyerWriteLine

indicates that the field needs to be compressed harder in order to

fit.  If this happens, the field should be closed and reopened using

a different level of compression or algorithm.  Also set the

FWF_REWRITE mode flag to indicate to replace the previous data.

When replacing fields in the middle of a clip, the compressed data

must be the same size or smaller, as no space insertion is currently

supported.  If an FERR_FULL occurs in this case, you must either

retry with a tighter compression method or write the original field

data back into the clip.  Otherwise, this field will flash

unpredictable data near the bottom when the clip is played back.

Always creates clips with integral color frames regardless of how

many fields are written.  If a clip is left with less than a full

color frame at the end, the remaining fields in the color frame are

temporariliy padded with NTSC black.  These pad fields are auto-

matically replaced when new fields are appended.

INPUTS

action - pointer to structure which describes a volume and the name

of the clip to operate on.

field - field number of clip (starts at 0).  Is a don't care with

some open modes (see below)

modes - flags describing how to handle writing field

FWF_NEW     - Erase existing clip (if any), start new clip

FWF_APPEND  - Append field to clip

FWF_REWRITE - Re-write over field

FWF_FRAME   - Re-write field (must redo all following fields in

the same color frame)

compinfo - pointer to a VidCompInfo structure (or null for defaults)

RESULT

EXAMPLE

NOTES

Replacing fields in the middle of clips not fully tested

BUGS

SEE ALSO

CloseField

EasyOpenWriteField

OpenReadField

FlyerWriteLine

flyer.library/PauseAction                           flyer.library/PauseAction

NAME

PauseAction - pause/resume a previously started action

SYNOPSIS

error = PauseAction(action,pauseflag)

D0                  A0     D0

ULONG PauseAction(struct ClipAction *,UBYTE pauseflag);

FUNCTION

Pauses or resumes a Flyer action that has been previously started.

Provide a pointer to the ClipAction structure used to start the

action.  No error occurs if action is already in the state specified

(already paused, for example).  AbortAction can be used to terminate

a paused action.

Does nothing if the action has already finished.

INPUTS

action - ptr to ClipAction structure used to start the action

pauseflag - 1 to pause, 0 to resume

RESULT

EXAMPLE

NOTES

Currently works only with FlyerRecord actions

BUGS

SEE ALSO

AbortAction

flyer.library/PlayMode                                 flyer.library/PlayMode

NAME

PlayMode - Ready Flyer for playback

SYNOPSIS

error = PlayMode(board)

D0               D0

ULONG PlayMode(UBYTE);

FUNCTION

Readies the Flyer for playback.  This takes about 1/2 second.

Return value indicates success (0) or the error code on failure

INPUTS

board - specifies the Flyer board (0-3)

RESULT

EXAMPLE

NOTES

You must currently ensure that no playing or recording is occurring

before calling this function

BUGS

SEE ALSO

flyer.library/PlaySequence                         flyer.library/PlaySequence

NAME

PlaySequence - Play the Flyer's internal sequence

SYNOPSIS

error = PlaySequence(board,basetime)

D0                   D0    D1

ULONG PlaySequence(UBYTE, ULONG);

FUNCTION

Starts the sequence playing that was previously downloaded to the

Flyer.  "basetime" is the time (on the Flyer's clock) to begin.  All

components in the sequence are relative to this start time.

This call returns immediately so that takes, effects may be done

synchronous with the sequence.  No other interaction with the Flyer

is required (or recommended) for the sequence to play, other than

aborting the sequence early (with AbortSequence).

See NewSequence for more info on Flyer sequencing.

INPUTS

board - specifies the Flyer board (0-3)

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

AddSeqClip

EndSequence

EndSequenceNew

NewSequence

flyer.library/PutFrameHeader                     flyer.library/PutFrameHeader

NAME

PutFrameHeader -- Write Frame Header structure back to clip

SYNOPSIS

error = PutFrameHeader(action,buffer)

D0                     A0     A1

ULONG PutFrameHeader(struct ClipAction *,APTR);

FUNCTION

Replaces a specific FrameHeader structure in an audio or video clip

with the data structure provided.  FrameHeader chosen is the one

that contains the field number specified in ca_VidStartField (even

for audio clips).

If the return value is not FERR_OKAY, something went wrong (such as

the clip was not found, or the requested field number is out of range).

CAUTION! This function is intended to be used to read/modify/write

a Frame Header (using GetFrameHeader).  It is dangerous if not

stupid to hand-craft a header from scratch and plug it in.  Doing

so is very difficult and is bound to cause problems.  Also, be very

cautious when modifying data in this structure.  The only thing

that's safe/useful to modify is the SerData buffer and associated

control values.  All else is toxic, flammable, noxious, etc.

INPUTS

clipaction - specifies the volume/clip name and the desired field number

buffer - Pointer to caller's structure to fill in

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

GetFrameHeader

flyer.library/Read10                                     flyer.library/Read10

NAME

Read10 -- Transfer data from SCSI drive to DMA memory

SYNOPSIS

error = Read10(action,blocks,lba,buffer)

D0             A0     D0     D1  D2

ULONG Read10(struct ClipAction *,WORD,ULONG,ULONG);

FUNCTION

INPUTS

clipaction - specifies the volume and return method

blocks - blocks to transfer

lba - starting lba

buffer - DMA buffer start (block) to receive data

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/ReadSize                                 flyer.library/ReadSize

NAME

ReadSize -- Read SCSI drive capacity

SYNOPSIS

error = ReadSize(flyervolume,countptr,lengthptr)

D0               A0          A1       A2

ULONG ReadSize(struct FlyerVolume *,ULONG *,ULONG *);

FUNCTION

INPUTS

volume - pointer to structure which specifies drive

countptr - pointer to ULONG to receive drive size in blocks

lengthptr - pointer to ULONG to receive logical block size (bytes)

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/ReadTest                                 flyer.library/ReadTest

NAME

ReadTest -- Do a read speed test on a Flyer SCSI drive

SYNOPSIS

error = ReadTest(flyervolume,blocks,repeat,lba,dblflag)

D0               A0          D0     D1     D2  D3

ULONG ReadTest(struct FlyerVolume *,ULONG,ULONG,ULONG,UBYTE);

FUNCTION

INPUTS

volume - pointer to structure which specifies drive

blocks - size of each transfer (in blocks)

repeat - number of transfers to perform

lba - starting lba on drive

dblflag - 0=simple test, 1=double-buffered test

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/RecordMode                             flyer.library/RecordMode

NAME

RecordMode - Ready Flyer for recording

SYNOPSIS

error = RecordMode(board)

D0                 D0

ULONG RecordMode(UBYTE);

FUNCTION

Readies the Flyer for recording.  This takes about 1/2 second.

Return value indicates success (0) or the error code on failure

INPUTS

board - specifies the Flyer board (0-3)

RESULT

EXAMPLE

NOTES

You must currently ensure that no playing or recording is occurring

before calling this function

BUGS

SEE ALSO

flyer.library/ReqSense                                 flyer.library/ReqSense

NAME

ReqSense -- Do SCSI RequestSense command

SYNOPSIS

error = ReqSense(flyervolume,buffersize,buffer)

D0               A0          D0         A1

ULONG ReqSense(struct FlyerVolume *,UBYTE,APTR);

FUNCTION

INPUTS

volume - pointer to structure which specifies drive

buffersize - size of data buffer provided (in bytes)

buffer - pointer to buffer to receive Sense data

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/ResetFlyer                             flyer.library/ResetFlyer

NAME

ResetFlyer -- Reset Flyer to known state

SYNOPSIS

error = ResetFlyer(board,flags)

D0                 D0    D1

ULONG ResetFlyer(UBYTE,ULONG);

FUNCTION

INPUTS

board - specifies the Flyer board (0-3)

flags - misc flags (unused)

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/SCSIinit                                 flyer.library/SCSIinit

NAME

SCSIinit -- Test and Initialize SCSI bus on Flyer

SYNOPSIS

error = SCSIinit(flyervolume)

D0               A0

ULONG SCSIinit(struct FlyerVolume *);

FUNCTION

INPUTS

volume - pointer to structure which specifies bus to init

RESULT

EXAMPLE

NOTES

Set v_SCSIdrive to the SCSI bus number x 8

Also set 'FVF_USENUMS' in v_Flags

BUGS

SEE ALSO

flyer.library/SCSIreset                               flyer.library/SCSIreset

NAME

SCSIreset -- Hardware reset all SCSI busses on Flyer

SYNOPSIS

error = SCSIreset(board)

D0                D0

ULONG SCSIreset(UBYTE);

FUNCTION

INPUTS

board - specifies the Flyer board (0-3)

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/SCSIseek                                 flyer.library/SCSIseek

NAME

SCSIseek -- Do SCSI seek command

SYNOPSIS

error = SCSIseek(flyervolume,lba)

D0               A0          D0

ULONG SCSIseek(struct FlyerVolume *,ULONG);

FUNCTION

INPUTS

volume - pointer to structure which specifies drive

lba - lba to which to seek

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/SetFillColor                         flyer.library/SetFillColor

NAME

SetFillColor - set fill color to use for blank video

SYNOPSIS

error = SetFillColor(action)

D0                   A0

ULONG SetFillColor(struct ClipAction *);

FUNCTION

Sets the specified Matte color as the fill color to use for blank

video, such as when skipping lines with SkipLines or closing the

write before all scan lines are transferred.

This color remains valid for the context of this field only.

Defaults to black when a new field is opened.

The fields which need setup prior to calling SetFillColor:

ca_FldHandle  - Field handle returned from successful

OpenReadField or (Easy)OpenWriteField call

ca_ReturnTime - RT_xxx value desired (not currently supported)

ca_MatteY     - Luminance value

ca_MatteI     - Signed I value

ca_MatteQ     - Signed Q value

INPUTS

action - pointer to structure which contains:

The field handle with which to associate this fill color

The fill color (in YIQ color space)

The return time for this call

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

CloseField

EasyOpenWriteField

OpenWriteField

FlyerWriteLine

flyer.library/SetFlooby                               flyer.library/SetFlooby

NAME

SetFlooby - used to set various Flyer internal values

SYNOPSIS

error = SetFlooby(board,chan,item,value)

D0                D0    D1   D2   D3

ULONG SetFlooby(UBYTE,UBYTE,UBYTE,ULONG);

FUNCTION

Selectively changes Flyer internal parameters by specifying the

parameter number and its new value.

INPUTS

board - specifies the Flyer board (0-3)

chan - video channel (0 or 1)

item - the parameter number to change

value - the value to assign to the parameter

RESULT

EXAMPLE

NOTES

All parameters are currently private.

Name derived from the term "FloobyDust"

BUGS

SEE ALSO

flyer.library/SetFlyerTime                         flyer.library/SetFlyerTime

NAME

SetFlyerTime - sets the Flyer's internal clock to a preset date/time

SYNOPSIS

error = SetFlyerTime(datestamp)

D0                   A0

ULONG SetFlyerTime(struct DateStamp *);

FUNCTION

Sets the internal real-time clock of all attached Flyers to the date

and time specified in the structure whose pointer is given.  They

maintain this date/time for the purpose of date-stamping files.

INPUTS

datestamp - pointer to an AmigaDOS DateStamp structure

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/SetSerDevice                         flyer.library/SetSerDevice

NAME

SetSerDevice - Select type of device attached to a Flyer serial port

SYNOPSIS

error = SetSerDevice(board,port,type,device)

D0                   D0    D1   D2   D3

ULONG SetSerDevice(UBYTE,UBYTE,UBYTE,UBYTE);

FUNCTION

Specifies the type and model of device which the user wishes to

attach to one of the Flyer's two serial ports.  These ports can be

used for a variety of things such as SMPTE read, SMPTE write, MIDI,

serial control, etc.  The Flyer will take care of details such as the

baud rate, format conversion, etc. for all devices defined in Flyer.h

INPUTS

board - specifies the Flyer board (0-3)

port - specifies the serial port (0 = A, 1 = B)

type - type of serial device (SERDEVTYPE_xxx in Flyer.h)

device - device model (SERDEV_xxx in Flyer.h)

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/SkipLines                               flyer.library/SkipLines

NAME

SkipLines - Seek past scan lines in a field previously opened

SYNOPSIS

error = SkipLines(action,lines)

D0                A0     D0

ULONG SkipLines(struct ClipAction *,UWORD);

FUNCTION

Seeks past a number of scan lines in a field previously opened.  If

opened for reading, skips over unwanted scan lines.  If opened for

writing, fills skipped lines with fillcolor (usually black).

Returns FERR_FULL if out of room in current field when writing.

The fields which need setup prior to calling SkipLines:

ca_FldHandle   - Field handle returned from successful

- OpenReadField or (Easy)OpenWriteField call

ca_ReturnTime  - RT_xxx value desired (not currently supported)

INPUTS

action - pointer to structure which contains the field handle to work

with and the return time for this call.

lines - number of scan lines to skip

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

CloseField

EasyOpenWriteField

OpenReadField

OpenWriteField

FlyerReadLine

SetFillColor

FlyerWriteLine

flyer.library/StartClipCutList                 flyer.library/StartClipCutList

NAME

StartClipCutList - Prepares to perform clip cutting

SYNOPSIS

error = StartClipCutList(clip,flags)

D0                       A0   D0

ULONG StartClipCutList(struct ClipAction *,UBYTE);

FUNCTION

Used to begin clip cutting and processing for the clip specified.

After opening the list with this function, use AddClipCut to make

each subclip definition, then close the list using EndClipCutList.

Two major types of processing can currently be accomplished using

this mechanism: destructive and non-destructive.  The destructive

processing will make the listed sub-clips and delete any unused parts

of the original, doing a regional de-frag operation so as to not

fragment the drive.  The non-destructive operation leaves the

original intact and makes new sub-clips.

INPUTS

clip - specifies the master clip from which to make sub-clip(s)

flags - specifies the type of processing (see Flyer.h CCL_xxx flags)

RESULT

EXAMPLE

NOTES

Currently only one ClipCut list may be open at a time.

BUGS

SEE ALSO

AddClipCut

EndClipCutList

flyer.library/StartHeadList                       flyer.library/StartHeadList

NAME

StartHeadList - Prepares Flyer for list of A/B heads

SYNOPSIS

error = StartHeadList(board)

D0                    D0

ULONG StartHeadList(UBYTE);

FUNCTION

Prepares specified Flyer to compose a list of A/B heads.

Create a list like this when opening an existing project.  This

will be more efficient than just submitting head definitions one

at a time, because it allows the Flyer to do some optimizations.

INPUTS

board - specifies the Flyer board (0-3)

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/StillMode                               flyer.library/StillMode

NAME

StillMode - Set video looping method for video channel

SYNOPSIS

error = StillMode(board,chan,mode)

D0                D0    D1   D2

ULONG StillMode(UBYTE,UBYTE,UBYTE);

FUNCTION

Used to specify the type of video looping to use on stilled video.

The default at power-up is MODE_FRAME.

INPUTS

volume - pointer to structure which describes a volume (used to

pick specific Flyer card).

chan - video channel (0 or 1)

mode - video looping mode:

MODE_FIELD - loops a single field of video

MODE_PAIR - loops an interlaced pair of video fields

MODE_FRAME - loops an entire color frame (default)

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/TBCcontrol                             flyer.library/TBCcontrol

NAME

TBCcontrol - Sense/control TBC functions

SYNOPSIS

error = TBCcontrol(board,TBCctrl,oper)

D0                 D0    A0      D1

ULONG TBCcontrol(UBYTE,struct TBCctrl *,UBYTE);

FUNCTION

Provides access to the (optional) Flyer TBC module.

The "oper" flags describe which portions of the TBCctrl structure to

apply.  This allows somewhat simplified use of this command without

always needing to set all values for each call, as well as the

ability to check the TBC status flags without modifying anything.

To determine if the TBC module is present, use this function setting

only the TBCOF_STATUS oper flag, then check the "status" flags

returned for TBCSF_MODULE to indicate that one was detected.

INPUTS

board - specifies the Flyer board (0-3)

TBCctrl - pointer to TBCctrl structure

oper - various flags indicating what kind of operation(s) to perform:

TBCOF_STATUS  -- get status flags

TBCOF_MODES   -- set modes, flags, and muxes

TBCOF_ADJUST  -- set adjustment values

Any combination of these operations can be specified.

RESULT

EXAMPLE

NOTES

If no TBC module is present, an error will be reported if this

command is used for anything except to get status

BUGS

SEE ALSO

FlyerInputSel

flyer.h

flyer.library/ToasterMux                             flyer.library/ToasterMux

NAME

ToasterMux - Set Flyer/Toaster multiplex switches

SYNOPSIS

error = ToasterMux(board,input3,input4,preview)

D0                 D0    D1     D2     D3

ULONG ToasterMux(UBYTE,UBYTE,UBYTE,UBYTE);

FUNCTION

Controls how the Flyer interacts with the Toaster's inputs 3 and 4

and preview output.

INPUTS

board - specifies the Flyer board (0-3)

input3 - video source fed to switcher input 3

0 = Toaster input 3

1 = Flyer video output (channel 0)

input4 - video source fed to switcher input 4

0 = Toaster input 4

1 = Flyer video output (channel 1)

preview - video fed to preview output

0 = Toaster preview bus

1 = Flyer camcorder input

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

FlyerInputSel

FlyerTermination

flyer.library/UnLockFlyVolList                 flyer.library/UnLockFlyVolList

NAME

UnLockFlyVolList - release lock on Flyer volumes list

SYNOPSIS

error = UnLockFlyVolList(list)

D0                       A0

ULONG UnLockFlyVolList(struct MinList *);

FUNCTION

Releases the lock obtained using LockFlyVolList.

Like most other library functions, a return value of FERR_OKAY

indicates success.

INPUTS

list - pointer to list (previously obtained with LockFlyVolList)

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

LockFlyVolList

flyer.library/VideoCompressModes             flyer.library/VideoCompressModes

NAME

VideoCompressModes - set video compression modes and strategy

SYNOPSIS

error = VideoCompressModes(board,bestmode,worstmode,strategy)

D0                         D0    D1       D2        D3

ULONG VideoCompressModes(UBYTE,UBYTE,UBYTE,UBYTE);

FUNCTION

Sets the range of video compression qualities that the Flyer may use

when recording video.  The default is all modes.  But this may be

pared down by narrowing this range or one specific mode may be

forced.

Strategy picks the strategy the Flyer should use for auto-switching

between modes.  The only supported value currently is 0, which uses

compressed data size to switch modes.

INPUTS

board - specifies the Flyer board (0-3)

bestmode - specifies the best video compression quality mode to use

worstmode - specifies the worst video compression quality mode to use

Currently supported modes, in order of decreasing video quality:

0 (D2)  Best quality, worst compression

1 (D2)

2 (SN)

3 (SN)

4 (SN)  Worst quality, best compression

strategy - always 0 for now (size based strategy)

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/VideoParams                           flyer.library/VideoParams

NAME

VideoParams - set video compression parameters

SYNOPSIS

err=VideoParams(board,vchan,mintol,maxtol,freq,vidlen,FIRset,special)

D0              D0    D1    D2     D3     D4   D5     D6     D7

ULONG VideoParams(UBYTE,UBYTE,UBYTE,UBYTE,UBYTE,UWORD,UBYTE,UBYTE);

FUNCTION

Sets the default video compression parameters for each video channel.

For auto-adjusting compression modes, this is only used for the first

field of video.

INPUTS

board - specifies the Flyer board (0-3)

vchan - video channel (0 or 1)

mintol - minimum tolerance mode (0 best, 6 worst)

maxtol - maximum tolerance mode

freq - random noise frequency

vidlen - desired length of compressed field data (in SCSI blocks)

FIRset - FIR presets to use

0 = custom

1 = 25%

2 = 33%

3 = 50%

4 = 100%

special - for testing only

RESULT

EXAMPLE

NOTES

Use only tolerance modes 0, 1, 4, 5, and 6

****** This may change as we finalize how the user's controls the

amount of compression ******

BUGS

SEE ALSO

flyer.library/VoidAllHeads                         flyer.library/VoidAllHeads

NAME

VoidAllHeads - Remove all A/B heads from all Flyers

SYNOPSIS

error = VoidAllHeads()

D0

ULONG VoidAllHeads(void);

FUNCTION

Removes all A/B heads from all drives attached to all Flyers.

INPUTS

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

MakeClipHead

VoidCardHeads

VoidClipHead

flyer.library/VoidCardHeads                       flyer.library/VoidCardHeads

NAME

VoidCardHeads - Remove all A/B heads for the Flyer card specified

SYNOPSIS

error = VoidCardHeads(board)

D0                    D0

ULONG VoidCardHeads(UBYTE board);

FUNCTION

Removes all A/B heads from drives attached to specified Flyer card

INPUTS

board - specifies the Flyer board (0-3)

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

MakeClipHead

VoidAllHeads

VoidClipHead

flyer.library/VoidClipHead                         flyer.library/VoidClipHead

NAME

VoidClipHead - Remove an A/B head for the specified clip

SYNOPSIS

error = VoidClipHead(clipaction)

D0                   A0

ULONG VoidClipHead(struct ClipAction *);

FUNCTION

Removes an A/B head for the specified clip.  Must exactly match the

range of a previously defined head (with MakeClipHead) or this

does nothing.

INPUTS

clipaction - specifies clip and in/out points of head to remove

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

MakeClipHead

VoidAllHeads

VoidCardHeads

flyer.library/WaitAction                             flyer.library/WaitAction

NAME

WaitAction - Wait for a previously issued action to complete

SYNOPSIS

error = WaitAction(action)

D0                 A0

ULONG WaitAction(struct ClipAction *);

FUNCTION

Does not return until the specified action is complete.

INPUTS

action - pointer to structure that was used to issue the original

command

RESULT

error - return code (from command)

EXAMPLE

NOTES

BUGS

SEE ALSO

CheckAction

flyer.library/Write10                                   flyer.library/Write10

NAME

Write10 -- Transfer data from DMA memory to SCSI drive

SYNOPSIS

error = Write10(action,blocks,buffer,lba)

D0              A0     D0     D1     D2

ULONG Write10(struct ClipAction *,WORD,ULONG,ULONG);

FUNCTION

INPUTS

clipaction - specifies the volume and the return method

blocks - blocks to transfer

buffer - DMA buffer start (block) of data to write

lba - starting lba

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

flyer.library/WriteTest                               flyer.library/WriteTest

NAME

WriteTest -- Do a write speed test on a Flyer SCSI drive

SYNOPSIS

error = WriteTest(flyervolume,blocks,repeat,lba,dblflag)

D0                A0          D0     D1     D2  D3

ULONG WriteTest(struct FlyerVolume *,ULONG,ULONG,ULONG,UBYTE);

FUNCTION

INPUTS

volume - pointer to structure which specifies drive

blocks - size of each transfer (in blocks)

repeat - number of transfers to perform

lba - starting lba on drive

dblflag - 0=simple test, 1=double-buffered test

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO